Faculty of Engineering & Technology

Department of Information and Communication Engineering

Course name : Signals and Systems Sessional

Course Code : ICE-2204

Lab Report On Signals and Systems

2nd Year 2nd Semester Department of ICE,PUST

INDEX

| Experiment no | Experiment name |
| --- | --- |
| 01 | Analysis of Discrete-Time Signals sequences: Impulse, Step, and Ramp Signals. |
| 02 | Operations on Discrete-Time Signals: Addition, Multiplication, Scaling, Shifting, and Folding. |
| 03 | Convolution of Discrete-Time Signals: Sinusoidal, Shifted, and Noisy Signals. |
| 04 | Auto-correlation and Cross-Correlation of Discrete- Time Signals. |
| 05 | Detecting peak and extracting feature from a PPG signal. |
| 06 | Fourier Series Approximation of a Square Wave. |
| 07 | Analyzing Fourier transform of a time function, for an aperiodic pulse. |
| 08 | Implementation of Discrete Fourier Transform (DFT) and Inverse Discrete Fourier Transform (IDFT). |
| 09 | Discrete Fourier Transform (DFT) and Frequency Bins Calculation. |

Experiment no : 1

Experiment name :	Analysis of Discrete-Time Signals sequences: Impulse, Step, and Ramp Signals.

Objectives :

- To understand and analyze fundamental discrete-time signals: impulse, step, and ramp signals.

- To visualize these signals using Python and Matplotlib.

- To comprehend the significance of these signals in signal processing and system analysis.

Theory :

In discrete-time signal processing, fundamental signals serve as the building blocks for analyzing and designing complex signals and systems. Three of the most basic discrete-time signals are the impulse signal, step signal, and ramp signal.

- Impulse Signal (δ[n])

The discrete-time impulse signal, also known as the unit impulse function or Kronecker delta function, is defined as:

This signal is used as an identity element in convolution and plays a crucial role in system analysis, as it helps determine a system’s impulse response.

- Step Signal (u[n])

The discrete-time unit step signal is defined as:

It represents a sudden transition from zero to one at n = 0. The step function is widely used in system analysis to represent signals that begin at a certain time and continue indefinitely.

- Ramp Signal (r[n])

The discrete-time ramp signal is defined as:

This signal is a linearly increasing function and is often used to model systems with constant acceleration or increasing growth trends.

Source Code :

import numpy as np

import matplotlib.pyplot as plt

#defining range

n = np.arange(-10, 11)

def impulse_signal(n):

return np.where(n == 0, 1, 0)

def step_signal(n):

return np.where(n >= 0, 1, 0)

def ramp_signal(n):

return np.where(n >= 0, n, 0)

# generating signals

impulse = impulse_signal(n) step = step_signal(n)

ramp = ramp_signal(n)

# plotting plt.figure(figsize=(12, 4))

plt.subplot(1, 3, 1) plt.stem(n, impulse) plt.title("Impulse Signal") plt.xlabel("n") plt.ylabel("Amplitude") plt.grid()

plt.subplot(1, 3, 2) plt.stem(n, step) plt.title("Step Signal") plt.xlabel("n") plt.ylabel("Amplitude") plt.grid()

plt.subplot(1, 3, 3) plt.stem(n, ramp) plt.title("Ramp Signal") plt.xlabel("n") plt.ylabel("Amplitude") plt.grid()

plt.tight_layout() plt.show()

Output :

Experiment no :	2

Experiment name :	Operations on Discrete-Time Signals: Addition, Multiplication, Scaling, Shifting, and Folding.

Objectives :

- To understand and analyze basic operations on discrete-time signals.

- To perform signal addition, multiplication, and scaling.

- To implement time-domain transformations such as shifting and folding.

- To visualize these operations using Python and Matplotlib.

Theory :

In digital signal processing, various operations on discrete-time signals help in system analysis and modification. Some fundamental operations include:

- Signal Addition

Signal addition involves summing the corresponding elements of two signals:

y[n] = x1[n] + x2[n]

This operation is useful in combining signals, such as in mixing audio signals.

- Signal Multiplication

Signal multiplication involves multiplying the corresponding elements of two signals:

y[n] = x1[n] * x2[n]

This operation is often used in modulation and feature extraction applications.

- Signal Scaling

Scaling a signal involves multiplying each value by a constant factor: y[n] = α * x[n]

This operation amplifies or attenuates the signal based on the scaling factor α.

- Signal Shifting (Time Shift)

Shifting a signal involves delaying or advancing its time indices:

  - Left Shift (Advance): y[n] = x[n+k](shifts the signal earlier).

  - Right Shift (Delay): y[n]=x[n−k] (shifts the signal later).

Shifting is commonly used in time-domain filtering and system response analysis.

- Signal Folding (Time Reversal)

Folding a signal flips it around the origin: y[n] = x[−n]

This operation is used in symmetry analysis and convolution operations.

Source Code :

import numpy as np

import matplotlib.pyplot as plt

# function for adding

def signal_addition(x1, x2): return x1 + x2

# function for multiplying

def signal_multiplication(x1, x2): return x1 * x2

# function for scaling

def signal_scaling(x, alpha): return alpha * x

# function for shifting

def signal_shifting(n, shift): return n + shift

# Function to fold (reverse) a signal def signal_folding(x):

return np.flip(x)

# signals

n = np.array([-2, -1, 0, 1, 2])

x1 = np.array([1, -2, 3, 4, 5])

x2 = np.array([5, 4, 3, 2, 1])

#operations

added_signal = signal_addition(x1, x2) multiplied_signal = signal_multiplication(x1, x2) scaled_signal = signal_scaling(x1, 2)

shifted_signal1 = signal_shifting(n, -2) # Shifted indices only shifted_signal2 = signal_shifting(n, +2) # Shifted indices only folded_signal = signal_folding(x1) # Folded indices

# plotting plt.figure(figsize=(12, 10))

plt.subplot(4, 2, 1) plt.stem(n, x1) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Original Signal x1") plt.grid()

plt.subplot(4, 2, 2) plt.stem(n, x2) plt.xlabel("Time ") plt.ylabel("Amplitude") plt.title("Original Signal x2") plt.grid()

plt.subplot(4, 2, 3) plt.stem(n, added_signal) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Signal Addition") plt.grid()

plt.subplot(4, 2, 4) plt.stem(n, multiplied_signal) plt.xlabel("Time") plt.ylabel("Amplitude")

plt.title("Signal Multiplication") plt.grid()

plt.subplot(4, 2, 5)

plt.stem(n, scaled_signal) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Scaled Signal (x1 * 2)") plt.grid()

plt.subplot(4, 2, 6) plt.stem(shifted_signal1, x1) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Shifted Signal (Shift = -2)") plt.grid()

plt.subplot(4, 2, 7) plt.stem(shifted_signal2,x1) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Shifted Signal (Shift = +2)") plt.grid()

plt.subplot(4, 2, 8) plt.stem(n, folded_signal) plt.xlabel("Time") plt.ylabel("Amplitude") plt.title("Folded Signal (x1)") plt.grid()

plt.tight_layout() plt.show()

Output :

Experiment no : 3

Experiment name :	Convolution of Discrete-Time Signals: Sinusoidal, Shifted, and Noisy Signals.

Objectives :

- To understand the concept of convolution in discrete-time signal processing.

- To perform and analyze the autoconvolution of a sinusoidal signal.

- To study the effect of convolution with a time-shifted version of a signal.

- To observe the impact of convolution when noise is introduced to a signal.

- To visualize convolution results using Python and Matplotlib.

Theory :

Convolution combines two functions (e.g., a signal and an impulse response) to produce a new function that represents their interaction.It is a fundamental operation in digital signal processing used for system analysis, filtering, and feature extraction. It is defined as:

where:

  - x1[n] and x2[n] are discrete signals.

  - y[n]is the convolution result.

- Autoconvolution of a Sinusoidal Signal

Autoconvolution involves convolving a signal with itself, which helps analyze signal energy distribution and periodicity.

- Convolution with a Shifted Version

When a signal is convolved with its shifted version, the phase relationship between the original and shifted signals affects the output. This process is useful in time-delay estimation and system identification.

- Convolution with a Noisy Signal

Introducing noise into a signal and performing convolution allows us to study the effects of noise filtering and signal enhancement techniques.

Source Code :

import numpy as np

import matplotlib.pyplot as plt from scipy.signal import convolve

# function for convolution

def compute_convolution(signal1, signal2):

conv_result = convolve(signal1, signal2, mode='full', method='auto') return conv_result

# frequency and time vector fs = 1000

t = np.linspace(0, 1, fs, endpoint=False) freq = 5 # frequency of the sine wave

#sinusoidal signal

sin_signal = np.sin(2 * np.pi * freq * t)

# 1: Autoconvolution of a sinusoidal signal

conv_auto = compute_convolution(sin_signal, sin_signal)

plt.figure(figsize=(12, 6)) plt.plot(conv_auto)

plt.title("Autoconvolution of a Sinusoidal Signal") plt.xlabel("Samples")

plt.ylabel("Convolution Output") plt.grid()

plt.show()

# 2: Convolution with a shifted version signal1 = sin_signal # sinusoidal signal

signal2 = np.roll(signal1, 100) # shifted version

conv_shifted = compute_convolution(signal1, signal2)

plt.figure(figsize=(12, 6)) plt.plot(conv_shifted)

plt.title("Convolution between Signal and Shifted Version") plt.xlabel("Samples")

plt.ylabel("Convolution Output")

plt.grid() plt.show()

# 3: Convolution with a noisy signal

noise = np.random.normal(0, 0.5, fs) #Gaussian noise noisy_signal = signal1 + noise

conv_noisy = compute_convolution(signal1, noisy_signal)

plt.figure(figsize=(12, 6)) plt.plot(conv_noisy) plt.title("Convolution with Noisy Signal") plt.xlabel("Samples") plt.ylabel("Convolution Output") plt.grid()

plt.show()

Output :

Experiment no : 4

Experiment name : Auto-correlation and Cross-Correlation of Discrete- Time Signals.

Objectives :

- To understand the concepts of auto-correlation and cross- correlation in signal processing.

- To compute and analyze the auto-correlation of a sinusoidal signal.

- To examine the cross-correlation between a signal and its time- shifted version.

- To evaluate the impact of noise on cross-correlation.

- To visualize these correlation functions using Python and Matplotlib.

Theory :

Correlation measures the similarity between two signals or functions. It quantifies how much one signal resembles another when shifted by a certain amount.

- Auto-correlation

Auto-correlation measures the similarity of a signal with a delayed version of itself. It helps in detecting periodicity and analyzing signal energy. The auto-correlation function is given by:

where:

  - Rx[k] is the auto-correlation function.

  - x[n] is the discrete-time signal.

  - k is the lag (shift) value.

Auto-correlation is widely used in applications such as radar, speech analysis, and system identification.

- Cross-Correlation

Cross-correlation measures the similarity between two different signals as a function of lag. It is useful in detecting time shifts and comparing signals. The cross-correlation function is given by:

where:

  - Rxy[k] is the cross-correlation function between x[n] and y[n].

  - k is the lag (shift) value.

Cross-correlation is commonly used in template matching, signal synchronization, and noise filtering.

- Cross-Correlation with a Noisy Signal

When noise is added to a signal, cross-correlation helps in extracting the desired signal from the noisy environment. This is useful in communication systems and data analysis.

Source code :

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import correlate, correlation_lags

#function for auto-correlation

def compute_autocorrelation(signal):

auto_corr = correlate(signal, signal, mode='full', method='auto') lags = correlation_lags(len(signal), len(signal), mode='full') return auto_corr, lags

#function for cross-correlation

def compute_cross_correlation(signal1, signal2):

cross_corr = correlate(signal1, signal2, mode='full', method='auto') lags = correlation_lags(len(signal1), len(signal2), mode='full') return cross_corr, lags

#frequency and time vector

fs = 1000

t = np.linspace(0, 1, fs, endpoint=False) freq = 5 #frequency of the sine wave

#sinusoidal signal

sin_signal = np.sin(2 * np.pi * freq * t)

# Compute and plot autocorrelation

auto_corr, lags = compute_autocorrelation(sin_signal) plt.figure(figsize=(12, 6))

plt.plot(lags, auto_corr)

plt.title("Autocorrelation of a Sinusoidal Signal") plt.xlabel("Lag")

plt.ylabel("Autocorrelation") plt.grid()

plt.show()

# cross-correlation between a signal and its shifted version

signal1 = sin_signal # sinusoidal signal

signal2 = np.roll(signal1, 100) # shifted version

cross_corr, lags = compute_cross_correlation(signal1, signal2) plt.figure(figsize=(12, 6))

plt.plot(lags, cross_corr)

plt.title("Cross-Correlation between Two Signals") plt.xlabel("Lag")

plt.ylabel("Cross-Correlation") plt.grid()

plt.show()

# cross-correlation with noise

noise = np.random.normal(0, 0.5, fs) #Gaussian noise noisy_signal = signal1 + noise

cross_corr_noise, lags = compute_cross_correlation(signal1, noisy_signal) plt.figure(figsize=(12, 6))

plt.plot(lags, cross_corr_noise)

plt.title("Cross-Correlation with Noisy Signal") plt.xlabel("Lag")

plt.ylabel("Cross-Correlation") plt.grid()

plt.show()

Output :

Experiment no : 5

Experiment name :	Detecting peak and extracting feature from a PPG signal.

Objectives :

- To generate a synthetic Photoplethysmogram (PPG) signal with noise.

- To apply a bandpass filter for noise reduction in the PPG signal.

- To detect peaks in the filtered signal corresponding to heartbeats.

- To estimate heart rate (HR) in beats per minute (BPM) from detected peaks.

- To visualize raw and processed PPG signals for analysis.

Theory :

- PPG Signal

Photoplethysmography (PPG) is a non-invasive optical technique used to measure blood volume changes in tissues. The periodic nature of the PPG waveform corresponds to the heartbeat.

- Bandpass Filtering

A Butterworth bandpass filter is applied to remove unwanted frequency components. The cutoff frequencies are:

  - 0.5 Hz (to remove low-frequency drift)

  - 5 Hz (to eliminate high-frequency noise)

The bandpass filter equation is:

where fc is the cutoff frequency and N is the filter order.

- Peak Detection

Peaks in the filtered PPG signal correspond to heartbeats. The scipy.signal.find_peaks function detects local maxima, ensuring a minimum distance of 50 samples between peaks to avoid false detections.

- Heart Rate Estimation

Heart rate (HR) is computed using:

where RR intervals are the time differences between consecutive detected peaks. The HR value is expressed in beats per minute (BPM).

- Signal Normalization

To improve peak detection, the filtered signal is normalized using:

This ensures the signal values are scaled between 0 and 1.

Source code :

import numpy as np

import scipy.signal as signal

import matplotlib.pyplot as plt

def bandpass_filter(data, fs=100):

b, a = signal.butter(4, [0.5 / (0.5 * fs), 5.0 / (0.5 * fs)], btype='band') return signal.filtfilt(b, a, data)

def detect_peaks(signal_data):

return signal.find_peaks(signal_data, distance=50)[0]

def detect_valleys(signal_data):

return signal.find_peaks(-signal_data, distance=50)[0]

def extract_heart_rate(peaks, fs=100): if len(peaks) < 2:

return 0

rr_intervals = np.diff(peaks) / fs return 60 / np.mean(rr_intervals)

# Generate synthetic PPG signal fs = 100

t = np.linspace(0, 10, fs * 10) sine_signal = np.sin(2 * np.pi * 1.2 * t)

noise_signal = 0.1 * np.random.normal(0, 1, len(t)) ppg_signal = sine_signal + noise_signal

# Process PPG signal

filtered_signal = bandpass_filter(ppg_signal, fs)

normalized_signal	=	(filtered_signal	-	np.min(filtered_signal))	/ (np.max(filtered_signal) - np.min(filtered_signal))

peaks = detect_peaks(normalized_signal) valleys = detect_valleys(normalized_signal) heart_rate = extract_heart_rate(peaks, fs)

# Print results

print("Filtered Signal (first 10 values):", filtered_signal[:10]) print("Detected Peaks (first 10 indices):", peaks[:10]) print("Detected Valleys (first 10 indices):", valleys[:10])

print(f"Estimated Heart Rate: {heart_rate:.2f} BPM")

# Plot results plt.figure(figsize=(12, 9))

plt.subplot(3, 2, 1)

plt.plot(t, sine_signal, label='Raw Sin Signal') plt.xlabel("Time")

plt.ylabel("Amplitude") plt.legend()

plt.subplot(3, 2, 2)

plt.plot(t, noise_signal, label='Raw Noise Signal') plt.xlabel("Time")

plt.ylabel("Amplitude") plt.legend()

plt.subplot(3, 2, 3)

plt.plot(t, ppg_signal, label='Raw PPG Signal')

plt.xlabel("Time") plt.ylabel("Amplitude") plt.legend()

plt.subplot(3, 2, 4)

plt.plot(t, filtered_signal, label='Filtered PPG Signal') plt.xlabel("Time")

plt.ylabel("Amplitude") plt.legend()

plt.subplot(3, 2, 5)

plt.plot(t, normalized_signal, label='Normalized PPG Signal') plt.xlabel("Time")

plt.ylabel("Amplitude") plt.legend()

plt.subplot(3, 2, 6)

plt.plot(t,	normalized_signal,label=f'PPG	with	Detected	Peaks(HR:

{heart_rate:.2f} BPM)')

plt.plot(t[peaks], normalized_signal[peaks],'ro', label='Detected Peaks') plt.plot(t[valleys],normalized_signal[valleys],'go',label='Detected Valleys')

plt.xlabel("Time") plt.ylabel("Amplitude") plt.legend() plt.tight_layout() plt.show()

Output :

Filtered Signal (first 10 values): [0.22942572 0.27059672 0.31263922

0.35637466	0.40244699	0.45125591	0.50291041	0.55720714

0.61363509 0.67140468]

Detected Peaks (first 10 indices): [ 20 104 186 271 353 436 518 604 685

771]

Detected Valleys (first 10 indices): [ 61 148 229 312 397 479 563 645

728 813]

Estimated Heart Rate: 71.90 BPM

Experiment no : 6

Experiment name : Fourier Series Approximation of a Square Wave.

Objectives :

- To approximate a square wave using Fourier series expansion.

- To analyze how the number of terms in the series affects the approximation.

- To visualize the Gibbs phenomenon in Fourier series representation.

Theory :

- Fourier Series Representation

A periodic function f(x) can be expressed as a sum of sine and cosine functions using the Fourier series:

For a square wave (odd function), only the sine terms contribute:

where n includes only odd harmonics.

- Gibbs Phenomenon

The Fourier series approximation of a discontinuous function (like a square wave) exhibits overshooting near the discontinuities. This

overshooting does not vanish even with more terms, a behavior known as the Gibbs phenomenon.

- Approximation with Finite Terms

The accuracy of the approximation improves as more terms are added:

  - 1 term: A single sine wave that roughly resembles the square wave.

  - 3 terms: A better approximation, but with visible oscillations.

  - 5 terms: Closer to the square wave, but still with overshooting.

  - 11 terms: Even closer, but Gibbs phenomenon persists.

Source Code :

import numpy as np

import matplotlib.pyplot as plt

# Fourier series approximation of a square wave def fourier_series(x, terms):

if terms < 1:

raise ValueError("Number of terms must be at least 1")

result = x - x

for n in range(1, terms + 1, 2): # Use only odd harmonics

result += (4 / (np.pi * n)) * np.sin(n * x) # Sum sine terms return result

# Define the original square wave function def square_wave(x):

return np.where(np.sin(x) >= 0, 1, -1) # Standard square wave

# Generate x values

t = np.linspace(-np.pi, np.pi, 400)

# Plot the original square wave plt.figure(figsize=(8, 6))

plt.plot(t, square_wave(t), label='Original Square Wave', linestyle='--', color='black')

for terms in [1, 3, 5, 11]:

plt.plot(t, fourier_series(t, terms), label=f'{terms} terms') plt.axhline(0, color='black', linewidth=0.5, linestyle='--') plt.title('Fourier Series Approximation of a Square Wave') plt.xlabel('Time')

plt.ylabel('Amplitude') plt.legend()

plt.grid() plt.show()

Output :

Experiment no : 7

Experiment name : Analyzing Fourier transform of a time function, for an aperiodic pulse.

Objectives :

- To	analyze	and	visualize	the	real,	phase,	and	magnitude components of a sinc function.

- To understand the behavior of the sinc function in both time and frequency domains.

- To	explore	the	significance	of	the	sinc	function	in	signal processing, particularly in filtering and reconstruction.

Theory :

The Fourier Transform is a mathematical operation that transforms a time-domain signal into its frequency-domain representation. It helps analyze how different frequency components contribute to the original signal.

For a continuous-time signal x(t), the Fourier Transform is defined as:

where:

  - X(f) is the Fourier Transform of x(t),

  - f is the frequency in Hertz (Hz),

−1

j =	represents the imaginary unit.

Let, an aperiodic pulse x(t) = 1, −2 < t <2 ,

Then,the Fourier transform X(f) :

.

By definition :

So, if x = 4f :

Now if we plot considering  ,	where t = 4πf

- Definition of the Sinc Function

The sinc function is defined as:

For the given function:

where the factor 4 scales the amplitude, and the argument 4t compresses the function along the time axis.

- Importance of the Sinc Function

  - The sinc function appears in Fourier transforms and is the impulse response of an ideal low-pass filter.

  - It is crucial in the Nyquist-Shannon sampling theorem, ensuring perfect signal reconstruction from discrete samples.

- Real, Phase, and Magnitude Components

  - Real Part: Represents the actual sinc waveform, oscillating and decaying over time.

  - Phase Part: Gives the angle (phase) of the function, which remains either 0 or π.

  - Magnitude	Part:	Shows	the	absolute	value	of	the	function, highlighting the envelope of sinc oscillations.

Source Code :

import numpy as np

import matplotlib.pyplot as plt

t = np.arange(-2, 2.01, 0.01) x = 4 * np.sinc(4 * t)

# Plot real part plt.figure(figsize=(10, 6))

plt.subplot(3, 1, 1) plt.plot(t, x) plt.xlabel('Time') plt.ylabel('Amplitude') plt.title('Real Part') plt.grid()

# Plot phase part plt.subplot(3, 1, 2)

plt.plot(t, np.angle(x)) # Angle (phase) of x plt.xlabel('Time')

plt.ylabel('Amplitude') plt.title('Phase Part') plt.grid()

# Plot magnitude part plt.subplot(3, 1, 3) plt.plot(t, np.abs(x)) plt.xlabel('Time') plt.ylabel('Amplitude') plt.title('Magnitude Part') plt.grid() plt.tight_layout() plt.show()

Output :

Experiment no : 8

Experiment name : Implementation of Discrete Fourier Transform (DFT) and Inverse Discrete Fourier Transform (IDFT).

Objectives :

- To compute the Discrete Fourier Transform (DFT) of a given input sequence.

- To reconstruct the original signal using the Inverse Discrete Fourier Transform (IDFT).

- To visualize the input sequence, its frequency components, and the reconstructed signal.

Theory :

Discrete Fourier Transform (DFT)

The Discrete Fourier Transform (DFT) is a mathematical tool used to analyze discrete-time signals in the frequency domain. It converts a sequence of N discrete values (time-domain) into another sequence of N complex values (frequency-domain).

For a discrete signal x[n] of length N, the DFT is defined as:

where:

  - X[k] is the DFT coefficient at frequency index k,

  - x[n] is the input sequence (time-domain signal),

  - N is the total number of samples,

−1

j =	(imaginary unit),

  - represents the complex exponential basis function.

Inverse Discrete Fourier Transform (IDFT)

The Inverse Discrete Fourier Transform (IDFT) converts the frequency- domain representation back into the time-domain sequence. It is given by:

where:

  - x[n] is the original time-domain signal,

  - X[k] is the DFT coefficient at frequency index k.

Now,the Discrete Fourier Transform (DFT) and the Inverse Discrete Fourier Transform (IDFT) for the sequence:

Compute the DFT :

The DFT formula for an N point sequence is:

where N=4, and k=0,1,2,3. For k=0:

For k = 1:

The exponential terms are:

which correspond to:

Thus:

For k = 2 :

The exponential terms are:

which correspond to:

Thus:

For k = 3:

The exponential terms are:

which correspond to:

Thus:

Final DFT Values :

Compute the IDFT :

The Inverse DFT is given by:

Since X[K] = {4,0,0,0} , we get:

for all n = 0,1,2,3. Final IDFT Values :

For visualize the DFT and IDFT we have to keep mind also :

Zero-Padding

If the length of the sequence is less than N, zero-padding is applied to extend the sequence. This helps in analyzing the frequency characteristics with better resolution.

Magnitude Spectrum

The magnitude spectrum is computed as:

Source Code :

import numpy as np

import matplotlib.pyplot as plt

# Input sequence and N x = [1,1,1,1]

N = 4

# Zero-padding if needed

x = np.pad(x, (0, N - len(x)), mode='constant')

# DFT computation X = np.fft.fft(x, N)

# IDFT computation (Inverse DFT) x_reconstructed = np.fft.ifft(X)

# Print the DFT and IDFT values print("DFT values:", X)

print("Reconstructed IDFT values:", x_reconstructed.real)

# Plot the input signal plt.figure(figsize=(10, 6))

plt.subplot(3, 1, 1) plt.stem(range(len(x)), x) plt.title('Input Signal x(n)') plt.xlabel('n')

plt.ylabel('x(n)')

plt.grid()

# Plot the magnitude of DFT plt.subplot(3, 1, 2) plt.stem(range(N), np.abs(X)) plt.title('DFT Magnitude |X(k)|') plt.xlabel('k')

plt.ylabel('|X(k)|') plt.grid()

# Plot the IDFT signal plt.subplot(3, 1, 3)

plt.stem(range(N), x_reconstructed.real) plt.title('Reconstructed Signal x(n) from IDFT') plt.xlabel('n')

plt.ylabel('x(n)') plt.grid() plt.tight_layout() plt.show()

Output :

Experiment no : 9

Experiment name :  Discrete Fourier Transform (DFT) and Frequency Bins Calculation.

Objective:

- To understand and implement the Discrete Fourier Transform (DFT) using Python.

- To compute and analyze the frequency bins associated with the DFT.

- To visualize the magnitude spectrum of the computed DFT.

- Theory:

1. Discrete Fourier Transform (DFT)

The Discrete Fourier Transform (DFT) is a mathematical technique used to analyze the frequency components of a discrete-time signal. It transforms a sequence of time-domain samples into its frequency-domain representation.

The formula for the DFT is:

where:

- X(k) represents the frequency-domain output,

- x(n) is the input time-domain signal,

- N is the total number of samples,

- k is the frequency index,

- is the complex exponential basis function.

2.Frequency Bins Calculation

The DFT outputs a set of frequency bins that represent discrete frequency components. The frequency bin formula is:

where:

- fk is the frequency of  bin k.

- Fs is the sampling frequency.

- N is the number of  DFT points.

Alternatively, we can compute both positive and negative frequency bins using NumPy’s :

This function generates frequency bins in the range:

which helps in analyzing both positive and negative frequencies.

- Source Code :

- import numpy as np

- import matplotlib.pyplot as plt

- def DFT(x):

- """Compute the Discrete Fourier Transform (DFT) of a 1D signal."""

- N = len(x)

- k = np.arange(N).reshape((N, 1))  # Column vector

- n = np.arange(N)  # Row vector

- exp_term = np.exp(-2j * np.pi * k * n / N)

- return np.dot(exp_term, x)

- # Sampling parameters

- Fs = 1000  # Sampling frequency (Hz)

- N = 1000   # Number of DFT points (same as samples)

- t = np.linspace(0, 1, N, endpoint=False)

- # Generate a signal with 50 Hz and 120 Hz sine waves

- signal = np.sin(2 * np.pi * 50 * t) + 0.5 * np.sin(2 * np.pi * 120 * t)

- # Compute DFT

- dft_output = DFT(signal)

- # Compute frequency bins using the formula f_k = (k / N) * Fs

- freq_bins = np.fft.fftfreq(N, d=1/Fs)

- # Plot single-sided magnitude spectrum

- plt.figure(figsize=(10, 5))

- plt.plot(freq_bins[:N//2], np.abs(dft_output[:N//2]))

- plt.title("DFT Frequency Spectrum")

- plt.xlabel("Frequency (Hz)")

- plt.ylabel("Magnitude")

- plt.grid()

- plt.show()

- Output :

- THE END