.INDEX.

| S.L. | Name of The Problem |
| --- | --- |
| 01 | # Plot the following signal operations using user defined function - a. adding ,b. multiplication, c. Scaling, d. shifting and e. folding. |
| 02 | Plot the following transformation x1(n) = 2x(n-5) - 3x(n+4). |
| 03 | Explain and Implementation the unit Impulse sequence, the unit Step sequence, the unit Ramp sequence. |
| 04 | Explain and Implement convolution of signal. |
| 05 | Explain and Implement correlation of signal. |
| 06 | Extract relevant features such as filtering, feature extraction, pick detection, heart rate etc. from PPG signal. |
| 07 | Explain and Implement Discrete Fourier Transform (DFT) using python. |
| 08 | Explain and Implement Frequency bin using python. |

# Problem No: 01

## Problem Name: Plot following signal operations using user defined function – a. adding, b. multiplication, c. Scaling, d. shifting and e. folding.

### Objective:

To implement and visualize basic signal operations (addition, multiplication, scaling, shifting, and folding) using user-defined functions in Python.

### Theory:

Signal processing involves various operations to manipulate signals for analysis and transformation. The fundamental operations are:

- Addition of Two Signals:
When two discrete-time signals x1[n] and x2[n] are added, the result is given by:

y[n]= x1​[n]+x2​[n]

This operation combines the amplitudes of both signals at corresponding time indices.

- Multiplication of Two Signals:
Element-wise multiplication of two signals is given by:

y(t) = x1(t) * x2(t)

This operation is used in modulation and filtering.

- Scaling of a Signal:
A signal x[n] is scaled by a factor α as follows:

y[n]=α⋅x[n]

This amplifies or attenuates the signal’s amplitude.

- Shifting of a Signal:
A signal is delayed or advanced by shifting its index by k:

  - Right shift (delay):

y[n]=x[n−k]

  - Left shift (advance):

y[n]=x[n+k]

- Folding (Time Reversal) of a Signal:
Folding reflects the signal around the vertical axis:

y[n]=x[−n]

This is useful in symmetry analysis and convolution.

### Source Code:

import numpy as np

import matplotlib.pyplot as plt

def signal_add(x1,x2):

return x1+x2

def signal_mul(x1,x2):

return x1*x2

def signal_scaling(x,alpha):

return alpha*x

def signal_shifting(n,shift):

return n+shift

def signal_folding(x):

return np.flip(x)

t = np.arange(-10,10,0.01)

x1 = np.array([1,2,3,4,5])

x2 = np.array([5,4,3,2,1])

n = np.array([-2,-1,0,1,2])

p1 = np.sin(2 * np.pi * 1 * t)

p2 = np.cos(2 * np.pi * 0.5 * t)

added_signal = signal_add(x1,x2)

multiplied_signal = signal_mul(x1,x2)

scaled_signal = signal_scaling(x1,2)

shifted_signal1 = signal_shifting(n,2)

shifted_signal2 = signal_shifting(n,-2)

folded_signal = signal_folding(x1)

plt.figure(figsize=(16,20))

plt.subplot(4,2,1)

plt.stem(n,x1)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('Original signal x1')

plt.grid()

plt.subplot(4,2,2)

plt.stem(n,x2)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('Original signal x2')

plt.grid()

plt.subplot(4,2,3)

plt.stem(n,added_signal)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('Signal addition')

plt.grid()

plt.subplot(4,2,4)

plt.stem(n,multiplied_signal)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('multiplied_signal')

plt.grid()

plt.subplot(4,2,5)

plt.stem(n,scaled_signal)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('Original signal (x1*2)')

plt.grid()

plt.subplot(4,2,6)

plt.stem(shifted_signal1,x1)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('2 shifted signal')

plt.grid()

plt.subplot(4,2,7)

plt.stem(shifted_signal2,x1)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('-2 shifted signal')

plt.grid()

plt.subplot(4,2,8)

plt.stem(n,folded_signal)

plt.xlabel("time")

plt.ylabel('amplitude')

plt.title('Folded signal(x1)')

plt.grid()

Sample Input & Output:

# Problem No: 02

## Problem Name: Plot the following transformation x1(n) = 2x(n-5) - 3x(n+4)

### Objective:

To plot the transformed signal x1(n) = 2x(n − 5) − 3x(n + 4) and visualize its effect on the original signal x(n).

### Theory:

A discrete-time signal x(n)x(n)x(n) can be modified using operations such as scaling, shifting, and multiplication.

The given transformation:

x1(n)=2x(n−5)−3x(n+4)

- Time Shifting:

  - x(n−5) shifts the signal right by 5 units (delayed).

  - x(n+4) shifts the signal left by 4 units (advanced).

- Scaling:

  - The signal x(n−5) is multiplied by 2.

  - The signal x(n+4) is multiplied by -3.

- Addition/Subtraction:

  - The final signal is obtained by adding the two scaled versions.

Source Code:

import numpy as np

import matplotlib.pyplot as plt

# Define the original signal x(n) (e.g., unit impulse or step function)

n = np.arange(-10, 10, 1)  # Discrete time range

x_n = np.zeros_like(n)  # Start with a zero array

# For simplicity, we will define x(n) as a unit impulse at n=0

x_n[n == 0] = 1  # Unit impulse at n=0

# Apply the transformation x1(n) = 2x(n-5) - 3x(n+4)

x1_n = 2 * np.roll(x_n, 5) - 3 * np.roll(x_n, -4)

# Plot the original signal and the transformed signal

plt.subplot(2, 1, 1)

plt.stem(n, x_n)

plt.title("Original Signal x(n)")

Text(0.5, 1.0, 'Original Signal x(n)')

plt.subplot(2, 1, 2)

plt.stem(n, x1_n)

plt.title("Transformed Signal x1(n) = 2x(n-5) - 3x(n+4)")

plt.tight_layout()

plt.show()

Sample Input & Output:

# Problem No: 03

## Problem Name: Explain and Implementation the unit Impulse sequence, the unit Step sequence, the unit Ramp sequence.

### Objective:

To generate and understand three fundamental signals: unit impulse, unit step, and unit ramp.

### Theory:

In signal processing and discrete-time systems, the Unit Impulse Sequence, Unit Step Sequence, and Unit Ramp Sequence are fundamental discrete-time signals used for analysis and system characterization.

Source Code:

import numpy as np

import matplotlib.pyplot as plt

# Generate a discrete-time index

n = np.arange(-10, 11)  # Range from -10 to 10

# Generate a random signal

random_signal = np.random.uniform(-1, 1, len(n))  # Random values between -1 and 1

# Step Sequence (u[n])

step_sequence = np.heaviside(n, 1)  # 1 for n >= 0, 0 for n < 0

# Impulse Sequence (δ[n])

impulse_sequence = np.zeros_like(n)  # All zeros

impulse_sequence[n == 0] = 1  # 1 at n = 0

# Ramp Sequence (r[n])

ramp_sequence = np.where(n >= 0, n, 0)  # n for n >= 0, else 0

# Plot the signals

plt.figure(figsize=(10, 8))

# Plot Random Signal

plt.subplot(4, 1, 1)

plt.stem(n, random_signal)

plt.title("Random Signal")

plt.xlabel("n")

plt.ylabel("Amplitude")

# Plot Step Sequence

plt.subplot(4, 1, 2)

plt.stem(n, step_sequence)

plt.title("Step Sequence u[n]")

plt.xlabel("n")

plt.ylabel("Amplitude")

# Plot Impulse Sequence

plt.subplot(4, 1, 3)

plt.stem(n, impulse_sequence)

plt.title("Impulse Sequence δ[n]")

plt.xlabel("n")

plt.ylabel("Amplitude")

# Plot Ramp Sequence

plt.subplot(4, 1, 4)

plt.stem(n, ramp_sequence)

plt.title("Ramp Sequence r[n]")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.tight_layout()

plt.show()

Sample Input & Output:

# Problem No: 04

## Problem Name: Explain and Implement convolution of signal.

### Objective:

To compute and visualize the convolution of two discrete-time signals, demonstrating how the output signal is formed by combining the input signal and the system's impulse response.

### Theory:

Convolution is a mathematical operation that combines two signals to form a third signal. It describes the output of a linear time-invariant (LTI) system when the input signal is passed through the system's impulse response.

Mathematically, discrete-time convolution is given by:

# Source Code:

import numpy as np

import matplotlib.pyplot as plt

# Generate two random signals

np.random.seed(42)  # Ensures reproducibility

x = np.random.randint(0, 10, 10)  # First random signal (length 10)

h = np.random.randint(0, 10, 5)   # Second random signal (length 5)

# Compute convolution (It combines two signals to produce a third signal)

y = np.convolve(x, h, mode='full')  # mode='full' returns the complete convolution result of length (N+M-1).

# Normalize convolution result

y_norm = y / np.max(np.abs(y))  # Normalize by maximum absolute value

# Define time indices

n_x = np.arange(len(x))  # Time indices for x

n_h = np.arange(len(h))  # Time indices for h

n_y = np.arange(len(y))  # Time indices for y

# Plot original signals and convolution result

plt.figure(figsize=(10, 6))

# Plot x[n]

plt.subplot(3, 1, 1)

plt.stem(n_x, x, linefmt='b-', markerfmt='bo', basefmt='k')

plt.title("Input Signal x[n]")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid()

# Plot h[n]

plt.subplot(3, 1, 2)

plt.stem(n_h, h, linefmt='r-', markerfmt='ro', basefmt='k')

plt.title("Filter/Impulse Response h[n]")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid()

# Plot normalized convolution result y[n]

plt.subplot(3, 1, 3)

plt.stem(n_y, y_norm, linefmt='g-', markerfmt='go', basefmt='k')

plt.title("Normalized Convolution Result y[n] = x[n] * h[n]")

plt.xlabel("n")

plt.ylabel("Normalized Amplitude")

plt.grid()

plt.tight_layout()

plt.show()

# Print values

print("Input Signal x[n]:", x)

print("Filter h[n]:", h)

print("Convolution Output y[n]:", y)

print("Normalized Convolution Output y[n]:", y_norm)

Sample Input & Output:

Input Signal x[n]: [6 3 7 4 6 9 2 6 7 4]

Filter h[n]: [3 7 7 2 5]

Convolution Output y[n]: [ 18  51  84  94 131 126 154 127 125 152  99  72  43  20]

Normalized Convolution Output y[n]: [0.11688312 0.33116883 0.54545455 0.61038961 0.85064935 0.81818182

1.         0.82467532 0.81168831 0.98701299 0.64285714 0.46753247

0.27922078 0.12987013]

# Problem No: 05

## Problem Name: Explain and Implement Correlation of signal.

### Objective:

To compute and visualize the correlation of signals, including both auto-correlation (a signal with itself) and cross-correlation (between two signals), to analyze their similarity and periodicity.

### Theory:

1. Cross-Correlation:

Cross-correlation measures the similarity between two different signals as one is shifted relative to the other. It helps in detecting patterns and aligning signals. Mathematically, cross-correlation is defined as:

Applications of Cross-Correlation:

- Signal synchronization in communication systems.

- Template matching in image processing.

- Feature detection in machine learning.

2. Auto-Correlation:

Auto-correlation measures the similarity of a signal with itself over different shifts. It helps in identifying repetitive patterns within a signal. Mathematically, it is given by:

Applications of Auto-Correlation:

- Speech processing: Identifying periodicity in audio signals.

- Radar and sonar: Detecting objects by analyzing echo patterns.

- DNA sequencing: Finding repeated gene patterns.

# Source Code:

import numpy as np

import matplotlib.pyplot as plt

# Create two signals

x = np.array([1, 2, 3, 4, 5])  # First signal

y = np.array([5, 4, 3, 2, 1])  # Second signal

# Compute Cross-Correlation (Cross-Correlation: Measures similarity between two different signals like x,y)

cross_corr = np.correlate(x, y, mode='full')

# Compute Auto-Correlation of x (Auto-Correlation: Measures similarity of a signal with itself )

auto_corr_x = np.correlate(x, x, mode='full')

# Compute Auto-Correlation of y

auto_corr_y = np.correlate(y, y, mode='full')

# Normalize Cross-Correlation

norm_factor = np.sqrt(np.correlate(x, x, mode='full')[len(x) - 1] * np.correlate(y, y, mode='full')[len(y) - 1])

cross_corr_norm = cross_corr / norm_factor

# Normalize Auto-Correlations

auto_corr_x_norm = auto_corr_x / auto_corr_x[len(x) - 1]  # Divide by max value

auto_corr_y_norm = auto_corr_y / auto_corr_y[len(y) - 1]  # Divide by max value

# Plot Cross-Correlation (Shows how x and y align at different shifts)

plt.figure(figsize=(10, 6))

plt.subplot(3, 1, 1)

plt.stem(range(-len(x) + 1, len(x)), cross_corr_norm)

plt.title("Normalized Cross-Correlation of x and y")

plt.xlabel("Lag")

plt.ylabel("Correlation (Normalized)")

# Plot Auto-Correlation of x

plt.subplot(3, 1, 2)

plt.stem(range(-len(x) + 1, len(x)), auto_corr_x_norm)

plt.title("Normalized Auto-Correlation of x")

plt.xlabel("Lag")

plt.ylabel("Correlation (Normalized)")

# Plot Auto-Correlation of y

plt.subplot(3, 1, 3)

plt.stem(range(-len(y) + 1, len(y)), auto_corr_y_norm)

plt.title("Normalized Auto-Correlation of y")

plt.xlabel("Lag")

plt.ylabel("Correlation (Normalized)")

plt.tight_layout()

plt.show()

# Print correlation values

print("Normalized Cross-Correlation:", cross_corr_norm)

print("Normalized Auto-Correlation of x:", auto_corr_x_norm)

print("Normalized Auto-Correlation of y:", auto_corr_y_norm)

Sample input & Output:

Normalized Cross-Correlation: [0.01818182 0.07272727 0.18181818 0.36363636 0.63636364 0.8

0.83636364 0.72727273 0.45454545]

Normalized Auto-Correlation of x: [0.09090909 0.25454545 0.47272727 0.72727273 1.         0.72727273

0.47272727 0.25454545 0.09090909]

Normalized Auto-Correlation of y: [0.09090909 0.25454545 0.47272727 0.72727273 1.         0.72727273

0.47272727 0.25454545 0.09090909]

# Problem No: 06

## Problem Name: Extract relevant features such as filtering, feature extraction, pick detection, heart rate etc. from PPG signal.

### Objective:

To extract relevant features such as filtering, peak detection, and heart rate calculation from a PPG (Photoplethysmogram) signal to analyze physiological parameters like heart rate and assess cardiovascular health.

### Theory:

1. PPG Signal:

- The Photoplethysmogram (PPG) signal is an optical measurement of blood volume changes in tissue.

- It is used to monitor heart rate (HR), blood oxygen levels (SpO₂), and vascular conditions.

- The signal consists of a periodic waveform, reflecting heartbeats, and may contain noise and artifacts.

2. Signal Processing Steps:

- Signal Generation:

  - A synthetic PPG signal is created using sinusoidal functions with added noise.

  - Option to introduce abnormalities such as irregular peaks for arrhythmia simulation.

- Normalization:

  - The signal is normalized using Min-Max Scaling:

  - Ensures all values lie between 0 and 1.

- Bandpass Filtering (Noise Removal):

  - A Butterworth bandpass filter is used to retain relevant heart rate frequencies.

  - Removes low-frequency drift and high-frequency noise.

- Peak Detection:

  - Identifies heartbeats by detecting peaks in the filtered signal.

  - Uses scipy.signal.find_peaks() with parameters such as minimum distance between peaks and prominence.

- Feature Extraction:

  - R-R Intervals (time between consecutive heartbeats) are computed.

  - Heart Rate (HR) is estimated as:

- Abnormality Detection:

  - Bradycardia: HR < 60 BPM (slow heart rate).

  - Tachycardia: HR > 100 BPM (fast heart rate).

  - Arrhythmia: Irregular heartbeat detected if R-R interval standard deviation is high.

# Source Code:

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import find_peaks, butter, filtfilt

# Simulating a synthetic PPG signal (for demonstration)

fs = 1000  # Sampling frequency (Hz)

t = np.linspace(0, 10, fs * 10)  # 10 seconds of data

ppg_signal = 0.5 * np.sin(2 * np.pi * 1.2 * t) + 0.3 * np.sin(2 * np.pi * 0.5 * t) + 0.1 * np.random.randn(len(t))

# Step 1: Bandpass Filtering (0.5-5 Hz)

def bandpass_filter(signal, lowcut, highcut, fs, order=4):

nyquist = 0.5 * fs

low = lowcut / nyquist

high = highcut / nyquist

b, a = butter(order, [low, high], btype='band')

filtered_signal = filtfilt(b, a, signal)

return filtered_signal

# Apply bandpass filter to PPG signal

lowcut = 0.5  # Low frequency (0.5 Hz)

highcut = 5.0  # High frequency (5 Hz)

filtered_ppg = bandpass_filter(ppg_signal, lowcut, highcut, fs)

# Step 2: Peak Detection

peaks, _ = find_peaks(filtered_ppg, distance=fs*0.6)  # Minimum distance between peaks (0.6 seconds)

# Step 3: Heart Rate Calculation

peak_intervals = np.diff(peaks) / fs  # Time between successive peaks (in seconds)

heart_rate = 60 / np.mean(peak_intervals)  # Heart rate in beats per minute (bpm)

# Plot the results

plt.figure(figsize=(12, 6))

# Original PPG Signal

plt.subplot(3, 1, 1)

plt.plot(t, ppg_signal, label='Raw PPG Signal')

plt.title('Raw PPG Signal')

plt.xlabel('Time (seconds)')

plt.ylabel('Amplitude')

plt.grid()

# Filtered PPG Signal

plt.subplot(3, 1, 2)

plt.plot(t, filtered_ppg, label='Filtered PPG Signal', color='orange')

plt.scatter(t[peaks], filtered_ppg[peaks], color='red', label='Detected Peaks')

plt.title('Filtered PPG Signal with Detected Peaks')

plt.xlabel('Time (seconds)')

plt.ylabel('Amplitude')

plt.grid()

# Heart Rate Information

plt.subplot(3, 1, 3)

plt.text(0.5, 0.5, f'Heart Rate: {heart_rate:.2f} bpm', fontsize=14, ha='center')

plt.axis('off')

plt.title('Heart Rate Information')

plt.tight_layout()

plt.show()

print(f"Estimated Heart Rate: {heart_rate:.2f} bpm")

Sample Input & Output:

# Problem No: 07

## Problem Name: Explain and Implement Discrete Fourier Transform (DFT) using python.

### Objective:

To explain and implement the Discrete Fourier Transform (DFT) using Python, and demonstrate how it can be used to analyze the frequency content of a discrete-time signal.

### Theory:

1. Discrete Fourier Transform (DFT):

DFT transforms a time-domain signal into its frequency-domain representation. It is mathematically defined as:

where:

- x[n] is the input signal.

- X[k] represents the frequency components of x[n].

- N is the total number of samples.

- k is the frequency index.

2. Inverse Discrete Fourier Transform (IDFT):

To recover the time-domain signal from its frequency representation, we use:

DFT is widely used for signal analysis, filtering, and frequency domain processing.

3. Implementation Details:

- The function DFT(x) computes the DFT from scratch using nested loops.

- A signal is generated with two frequency components (70 Hz and 90 Hz).

- The DFT is applied to analyze the frequency spectrum.

- The magnitude spectrum is plotted for interpretation.

Source Code:

import numpy as np

import matplotlib.pyplot as plt

# Step 1: Create a sample discrete signal (for example, a sum of two sinusoids)

fs = 1000  # Sampling frequency (Hz)

t = np.arange(0, 1, 1/fs)  # Time vector for 1 second

f1 = 50  # Frequency of first sinusoid (Hz)

f2 = 120  # Frequency of second sinusoid (Hz)

# Create a signal that is the sum of two sinusoids

x = np.sin(2 * np.pi * f1 * t) + np.sin(2 * np.pi * f2 * t)

# Step 2: Compute the DFT of the signal manually using the DFT formula

N = len(x)  # Length of the signal

X = np.zeros(N, dtype=complex)  # Initialize the DFT array

# Calculate the DFT using the DFT formula

for k in range(N):

sum_val = 0

for n in range(N):

sum_val += x[n] * np.exp(-2j * np.pi * k * n / N)

X[k] = sum_val

# Step 3: Compute the frequency axis for plotting

frequencies = np.fft.fftfreq(N, 1/fs)  # Frequencies corresponding to DFT bins

# Step 4: Plot the original signal and the magnitude of the DFT

plt.figure(figsize=(12, 6))

# Plot the original signal

plt.subplot(2, 1, 1)

plt.plot(t, x)

plt.title('Original Signal (Sum of Two Sinusoids)')

plt.xlabel('Time (seconds)')

plt.ylabel('Amplitude')

plt.grid()

# Plot the magnitude of the DFT (frequency domain representation)

plt.subplot(2, 1, 2)

plt.plot(frequencies[:N//2], np.abs(X)[:N//2])  # Plot only the positive frequencies

plt.title('Magnitude of DFT (Frequency Domain)')

plt.xlabel('Frequency (Hz)')

plt.ylabel('Magnitude')

plt.grid()

plt.tight_layout()

plt.show()

Sample Input & Output:

# Problem No: 08

## Problem Name: Explain and Implement Frequency bin using python.

### Objective:

To understand and implement the Discrete Fourier Transform (DFT) and its frequency bins, and to analyze the frequency content of a discrete-time signal using Python.

### Theory:

1. Fast Fourier Transform (FFT):

FFT is an efficient algorithm to compute the Discrete Fourier Transform (DFT). It converts a time-domain signal into the frequency domain using:

where:

- x[n] is the input signal.

- X[k] represents its frequency components.

- N is the number of samples.

2. Frequency Filtering in FFT Domain:

- High-frequency components above 500 Hz are removed by setting their magnitude to zero in the frequency spectrum.

- The cleaned signal is reconstructed using the Inverse FFT (IFFT):

3. Generating a Noisy Signal:

- A pure sinusoidal wave at 440 Hz (A4 note in music) is created.

- Random noise is added to simulate real-world signal corruption.

- FFT filtering helps remove noise and restore the original signal.

Source Code:

import numpy as np

import matplotlib.pyplot as plt

from scipy.fft import fft, ifft, fftfreq

# Parameters

Fs = 1000  # Sampling frequency

T = 1 / Fs  # Sampling interval

t = np.linspace(0, 1, Fs, endpoint=False)  # Time vector

# Generate signals

pure_signal = np.sin(2 * np.pi * 440 * t)  # 440 Hz sine wave

noisy_signal = pure_signal + np.random.normal(0, 0.5, t.shape)  # Add noise

# Apply FFT and filter

fft_signal = fft(noisy_signal)

freqs = fftfreq(len(fft_signal), T)

fft_signal[np.abs(freqs) > 500] = 0  # Remove frequencies > 500 Hz

cleaned_signal = ifft(fft_signal).real  # Inverse FFT to get cleaned signal

# Plotting

plt.figure(figsize=(12, 6))

plt.subplot(3, 1, 1)

plt.plot(t, pure_signal, 'k', label="Original Signal")

plt.subplot(3, 1, 2)

plt.plot(t, noisy_signal, 'r', label="Noisy Signal")

plt.subplot(3, 1, 3)

plt.plot(t, cleaned_signal, 'y', label="Cleaned Signal")

plt.tight_layout()

plt.show()

Sample Input & Output: