Department of

INFORMATION AND COMMUNICATION ENGINEERING

PABNA UNIVERSITY OF SCIENCE AND TECHNOLOGY

Submitted To :

Dr. Md. Imran Hossain

Associate Professor

Dept. of Information and Communication Engineering

Submitted By :

Md.Sabbir Hossain

Roll : 220603

Session : 2021-22

Dept. of Information and Communication Engineering

LAB REPORT

ICE-2204
Signals and systems Sessional

Submission Date : 03-03-2025

Signals And Systems Lab Report

Experiment No-1:

Tittle:Visualization of Discrete-Time Signals: (Impulse,Step,and Ramp Signals )

Theory:

  Impulse Signal (δ[n]): A discrete-time signal that is 1 at n = 0 and 0 elsewhere.

δ[n]={1if n=00if n≠0\delta[n] = \begin{cases} 1 & \text{if } n = 0 \\ 0 & \text{if } n \neq 0 \end{cases}δ[n]={10​if n=0if n=0​

  Step Signal (u[n]): A signal that is 0 for n < 0 and 1 for n ≥ 0.

u[n]={0if n<01if n≥0u[n] = \begin{cases} 0 & \text{if } n < 0 \\ 1 & \text{if } n \geq 0 \end{cases}u[n]={01​if n<0if n≥0​

  Ramp Signal (r[n]): A signal that increases linearly with n for n ≥ 0 and is 0 for n < 0.

r[n]={nif n≥00if n<0r[n] = \begin{cases} n & \text{if } n \geq 0 \\ 0 & \text{if } n < 0 \end{cases}r[n]={n0​if n≥0if n<0​

Objectives:

The goal of this experiment is to generate and analyze three fundamental discrete-time signals: Impulse, Step, and Ramp signals, and visualize their behavior.

Code:

import numpy as np

import matplotlib.pyplot as plt

# Define the range

n = np.arange(-10, 11)

def impulse_signal(n):

return np.where(n == 0, 1, 0)

def step_signal(n):

return np.where(n >= 0, 1, 0)

def ramp_signal(n):

return np.where(n >= 0, n, 0)

# Generate signals

impulse = impulse_signal(n)

step = step_signal(n)

ramp = ramp_signal(n)

# Plot signals

plt.figure(figsize=(12, 4))

plt.subplot(1, 3, 1)

plt.stem(n, impulse)

plt.title("Impulse Signal")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid()

plt.subplot(1, 3, 2)

plt.stem(n, step)

plt.title("Step Signal")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid()

plt.subplot(1, 3, 3)

plt.stem(n, ramp)

plt.title("Ramp Signal")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid()

plt.tight_layout()

plt.show()

Input:

Output :

Purpose:

The purpose of this lab is to:

- Understand the properties and definitions of impulse, step, and ramp signals.

- Use Python to generate these signals over a range of nnn values.

- Visualize the signals using plots to analyze their behaviors.

Experiment No-02:

Tittle: Signal Operations: Addition, Multiplication, Scaling, Shifting, and Folding

Theory:

Signal processing involves manipulating signals to modify or enhance them. Common operations include:

- Addition: Combining two signals element-wise.

- Multiplication: Element-wise product of two signals.

- Scaling: Multiplying a signal by a constant to change its amplitude.

- Shifting: Translating a signal along the time axis.

- Folding: Reversing the signal with respect to time.

These operations are essential in fields like communications, audio processing, and control systems.

Objectives:

  Learn basic signal operations (addition, multiplication, scaling, shifting, folding).

  Visualize the effects of these operations on signals.

  Understand real-world applications of these operations in signal processing.

  Practice using Python for signal manipulation and visualization.

Code:

import numpy as np

import matplotlib.pyplot as plt

def signal_addition(x1, x2):

return x1 + x2

def signal_multiplication(x1, x2):

return x1 * x2

def signal_scaling(x, alpha):

return alpha * x

def signal_shifting(n, shift):

return n + shift

def signal_folding(x):

return np.flip(x)

n = np.array([-2, -1, 0, 1, 2])

x1 = np.array([1, 2, 3, 4, 5])

x2 = np.array([5, 4, 3, 2, 1])

added_signal = signal_addition(x1, x2)

multiplied_signal = signal_multiplication(x1, x2)

scaled_signal = signal_scaling(x1, 2)

shifted_signal1 = signal_shifting(n, -2)

shifted_signal2 = signal_shifting(n, 2)

folded_signal = signal_folding(x1)

plt.figure(figsize=(12, 10))

plt.subplot(4, 2, 1)

plt.stem(n, x1)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Original Signal x1")

plt.grid()

plt.subplot(4, 2, 2)

plt.stem(n, x2)

plt.xlabel("Time ")

plt.ylabel("Amplitude")

plt.title("Original Signal x2")

plt.grid()

plt.subplot(4, 2, 3)

plt.stem(n, added_signal)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Signal Addition")

plt.grid()

plt.subplot(4, 2, 4)

plt.stem(n, multiplied_signal)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Signal Multiplication")

plt.grid()

plt.subplot(4, 2, 5)

plt.stem(n, scaled_signal)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Scaled Signal (x1 * 2)")

plt.grid()

plt.subplot(4, 2, 6)

plt.stem(shifted_signal1, x1)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Shifted Signal (Shift = -2)")

plt.grid()

plt.subplot(4, 2, 7)

plt.stem(shifted_signal2, x1)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Shifted Signal (Shift = +2)")

plt.grid()

plt.subplot(4, 2, 8)

plt.stem(n, folded_signal)

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.title("Folded Signal (x1)")

plt.grid()

plt.tight_layout()

plt.show()

Input:

Output :

Purpose:

The purpose of the program is to demonstrate common signal operations, including signal addition, multiplication, scaling, shifting, and folding, and visualize their effects on the input signals.

Experiment No-03:

Tittle: Autocorrelation and Cross-Correlation Analysis of Signals with Noise

Theory:

- Autocorrelation measures the similarity of a signal with itself over different time lags, helping to identify periodicity or patterns within the signal.

- Cross-correlation measures the similarity between two signals, helping to detect relationships and align signals in time.

These techniques are used in signal detection, noise reduction, and time delay estimation.

Objectives:

- Understand and compute autocorrelation and cross-correlation.

- Analyze signal periodicity and the relationship between signals.

- Visualize how correlation can help in real-world signal processing tasks.

- Learn to implement these methods using Python libraries.

Code:

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import correlate, correlation_lags

def compute_autocorrelation(signal):

auto_corr = correlate(signal, signal, mode='full', method='auto')

lags = correlation_lags(len(signal), len(signal), mode='full')

return auto_corr, lags

def compute_cross_correlation(signal1, signal2):

cross_corr = correlate(signal1, signal2, mode='full', method='auto')

lags = correlation_lags(len(signal1), len(signal2), mode='full')

return cross_corr, lags

fs = 1000  # Sampling frequency in Hz

t = np.linspace(0, 1, fs, endpoint=False)  # Time vector

freq = 5  # Frequency of the sine wave

sin_signal = np.sin(2 * np.pi * freq * t)

auto_corr, lags_auto = compute_autocorrelation(sin_signal)

signal1 = sin_signal

signal2 = np.roll(signal1, 100)

cross_corr, lags_cross = compute_cross_correlation(signal1, signal2)

noise = np.random.normal(0, 0.5, fs)

noisy_signal = signal1 + noise

cross_corr_noise, lags_noise = compute_cross_correlation(signal1, noisy_signal)

plt.figure(figsize=(12, 12))

plt.subplot(3, 1, 1)

plt.plot(lags_auto, auto_corr)

plt.title("Autocorrelation of a Sinusoidal Signal")

plt.xlabel("Lag")

plt.ylabel("Autocorrelation")

plt.grid()

plt.subplot(3, 1, 2)

plt.plot(lags_cross, cross_corr)

plt.title("Cross-Correlation between Two Signals")

plt.xlabel("Lag")

plt.ylabel("Cross-Correlation")

plt.grid()

plt.subplot(3, 1, 3)

plt.plot(lags_noise, cross_corr_noise)

plt.title("Cross-Correlation with Noisy Signal")

plt.xlabel("Lag")

plt.ylabel("Cross-Correlation")

plt.grid()

plt.tight_layout()

plt.show()

Input:

The input consists of:

- A sinusoidal signal with a frequency of 5 Hz, sampled at 1000 Hz.

- A shifted version of the sinusoidal signal for cross-correlation.

- A noisy version of the sinusoidal signal (created by adding random noise).

Output:

Purpose:

The purpose of this program is to compute and visualize:

- The autocorrelation of a sinusoidal signal.

- The cross-correlation between two signals (one being a shifted version of the other).

- The cross-correlation between a sinusoidal signal and a noisy signal, to observe how noise affects the correlation.

Experiment No: 04

Tittle: Convolution Analysis of Signals: Autoconvolution, Shifted Signals, and Noise Effects

Theory:

Convolution is a mathematical operation that combines two signals to produce a third, showing their overlap as a function of time shift. Key concepts include:

- Autoconvolution: Convolving a signal with itself to detect patterns and periodicity.

- Convolution with Shifted Signals: Analyzing the similarity between a signal and its shifted version.

- Convolution with Noisy Signals: Studying the effect of noise on a signal through convolution.

Objectives:

  Understand and visualize autoconvolution and its properties.

  Analyze convolution with shifted signals to measure time shifts and similarity.

  Investigate the effect of noise in signal processing through convolution.

  Practice implementing convolution using Python and relevant libraries.

Code:

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import convolve

def compute_convolution(signal1, signal2):

conv_result = convolve(signal1, signal2, mode='full', method='auto')

return conv_result

fs = 1000  # Sampling frequency in Hz

t = np.linspace(0, 1, fs, endpoint=False)  # Time vector

freq = 5  # Frequency of the sine wave

sin_signal = np.sin(2 * np.pi * freq * t)

conv_auto = compute_convolution(sin_signal, sin_signal)

signal1 = sin_signal

signal2 = np.roll(signal1, 100)

conv_shifted = compute_convolution(signal1, signal2)

noise = np.random.normal(0, 0.5, fs)

noisy_signal = signal1 + noise

conv_noisy = compute_convolution(signal1, noisy_signal)

plt.figure(figsize=(12, 12))

plt.subplot(3, 1, 1)

plt.plot(conv_auto)

plt.title("Autoconvolution of a Sinusoidal Signal")

plt.xlabel("Samples")

plt.ylabel("Convolution Output")

plt.grid()

plt.subplot(3, 1, 2)

plt.plot(conv_shifted)

plt.title("Convolution between Signal and Shifted Version")

plt.xlabel("Samples")

plt.ylabel("Convolution Output")

plt.grid()

plt.subplot(3, 1, 3)

plt.plot(conv_noisy)

plt.title("Convolution with Noisy Signal")

plt.xlabel("Samples")

plt.ylabel("Convolution Output")

plt.grid()

plt.tight_layout()

plt.show()

Input:

The input consists of:

- A sinusoidal signal with a frequency of 5 Hz, sampled at 1000 Hz.

- A shifted version of the sinusoidal signal for convolution.

- A noisy version of the sinusoidal signal (created by adding random noise)

Output :

Purpose:

The purpose of this program is to compute and visualize:

- The autoconvolution of a sinusoidal signal.

- The convolution between a sinusoidal signal and its shifted version.

- The convolution between a sinusoidal signal and a noisy signal, to observe the effect of noise on the convolution output.

Experiment No- 05

Tittle - Heart Rate Estimation from PPG Signal Using Bandpass Filtering and Peak Detection

Theory:

This code processes a PPG signal to estimate heart rate by:

- Bandpass Filtering: Removes noise and retains frequencies relevant to the heart rate (0.5 Hz to 5 Hz).

- Peak Detection: Identifies peaks in the signal corresponding to heartbeats.

- Heart Rate Estimation: Calculates the heart rate by averaging the time intervals between peaks (R-R intervals).

- Normalization: Scales the signal for easier peak detection.

Objectives:

  Apply bandpass filtering to clean the PPG signal.

  Detect peaks in the filtered signal.

  Estimate heart rate by analyzing the R-R intervals.

  Visualize the signal processing stages.

  Understand how signal processing techniques are used in health monitoring for heart rate estimation.

Code:

import numpy as np

import scipy.signal as signal

import matplotlib.pyplot as plt

def bandpass_filter(data, fs=100):

b, a = signal.butter(4, [0.5 / (0.5 * fs), 5.0 / (0.5 * fs)], btype='band')

return signal.filtfilt(b, a, data)

def detect_peaks(signal_data):

return signal.find_peaks(signal_data, distance=50)[0]

def extract_heart_rate(peaks, fs=100):

if len(peaks) < 2:

return 0

rr_intervals = np.diff(peaks) / fs

return 60 / np.mean(rr_intervals)

# Generate synthetic PPG signal

fs = 100

t = np.linspace(0, 10, fs * 10)

sine_signal = np.sin(2 * np.pi * 1.2 * t)

noise_signal = 0.1 * np.random.normal(0, 1, len(t))

ppg_signal = sine_signal + noise_signal

# Process PPG signal

filtered_signal = bandpass_filter(ppg_signal, fs)

normalized_signal = (filtered_signal - np.min(filtered_signal)) / (np.max(filtered_signal) -

np.min(filtered_signal))

peaks = detect_peaks(normalized_signal)

heart_rate = extract_heart_rate(peaks, fs)

# Print results

print("Filtered Signal (first 10 values):", filtered_signal[:10])

print("Detected Peaks (first 10 indices):", peaks[:10])

print(f"Estimated Heart Rate: {heart_rate:.2f} BPM")

# Plot results

plt.figure(figsize=(12, 9))

plt.subplot(3, 2, 1)

plt.plot(t, sine_signal, label='Raw Sine Signal')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.subplot(3, 2, 2)

plt.plot(t, noise_signal, label='Raw Noise Signal')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.subplot(3, 2, 3)

plt.plot(t, ppg_signal, label='Raw PPG Signal')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.subplot(3, 2, 4)

plt.plot(t, filtered_signal, label='Filtered PPG Signal')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.subplot(3, 2, 5)

plt.plot(t, normalized_signal, label='Normalized PPG Signal')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.subplot(3, 2, 6)

plt.plot(t, normalized_signal,label=f'PPG with Detected Peaks')

plt.plot(t[peaks], normalized_signal[peaks],'ro', label='Detected Peaks')

plt.xlabel("Time")

plt.ylabel("Amplitude")

plt.legend()

plt.tight_layout()

plt.show()

Input:

The input consists of:

- A synthetic PPG signal, created by adding a sine wave of 1.2 Hz frequency and Gaussian noise.

- A sampling frequency fs=100fs = 100fs=100 Hz.

Output:

Purpose: The purpose of this program is to:

- Filter the PPG signal using a bandpass filter.

- Normalize the filtered signal for peak detection.

- Detect peaks in the PPG signal.

- Estimate heart rate based on the detected peaks by calculating the RR intervals and computing the average heart rate in beats per minute (BPM).

Experiment No: 06

Tittle: Discrete Fourier Transform (DFT) and Inverse DFT (IDFT) of a Signal

Theory:

- DFT (Discrete Fourier Transform) converts a time-domain signal into its frequency components, revealing the signal's frequency content.

- IDFT (Inverse Discrete Fourier Transform) reconstructs the original time-domain signal from its frequency components.

These transforms are essential in signal processing for tasks like spectrum analysis and signal reconstruction.

Objectives:

  Compute the DFT of a signal and analyze its frequency components.

  Reconstruct the signal using the IDFT.

  Visualize the original signal, its frequency spectrum, and the reconstructed signal.

Code:

mport numpy as np

import matplotlib.pyplot as plt

# Input sequence and N

x = [1,1,1,1]

N= 4

x = np.pad(x, (0, N - len(x)), mode='constant')

# DFT computation

X = np.fft.fft(x, N)

# IDFT computation (Inverse DFT)

x_reconstructed = np.fft.ifft(X)

# Print the DFT and IDFT values

print("DFT values:", X)

print("Reconstructed IDFT values:", x_reconstructed.real)

# Plot the input signal

plt.figure(figsize=(10, 6))

plt.subplot(3, 1, 1)

plt.stem(range(len(x)), x)

plt.title('Input Signal x(n)')

plt.xlabel('n')

plt.ylabel('x(n)')

plt.grid()

# Plot the magnitude of DFT

plt.subplot(3, 1, 2)

plt.stem(range(N), np.abs(X))

plt.title('DFT Magnitude |X(k)|')

plt.xlabel('k')

plt.ylabel('|X(k)|')

plt.grid()

# Plot the IDFT signal

plt.subplot(3, 1, 3)

plt.stem(range(N), x_reconstructed.real)

plt.title('Reconstructed Signal x(n) from IDFT')

plt.xlabel('n')

plt.ylabel('x(n)')

plt.grid()

plt.tight_layout()

plt.show()

Input:

The input consists of:

- A sequence x=[1,1,1,1]

- The length N=4 for the DFT computation.

Output:

Purpose:

The purpose of this program is to:

- Compute the Discrete Fourier Transform (DFT) of the input signal.

- Reconstruct the original signal using the Inverse Discrete Fourier Transform (IDFT).

- Visualize the input signal, the magnitude of the DFT, and the reconstructed signal.

Experiment No: 07

Tittle : Fourier Series Approximation of a Square Wave

Theory:

The Fourier Series is a mathematical tool used to express a periodic function as a sum of sine and cosine functions. It helps to approximate complex periodic signals using simpler sinusoidal components.

- Square Wave:

  - A square wave is a periodic waveform that alternates between two levels, typically +1 and -1. It is a non-sinusoidal waveform, often used in digital signals and communications.

- Fourier Series Approximation:

  - The Fourier series allows us to approximate any periodic function (such as a square wave) as a sum of sinusoidal functions (sines and cosines). For a square wave, the Fourier series consists only of odd harmonics (sine terms with odd multiples of the fundamental frequency).

  - As the number of terms in the Fourier series increases, the approximation becomes closer to the original square wave, exhibiting sharp transitions between the levels.

Objectives:

  Understand Fourier Series:

- Learn how Fourier series decompose a periodic function into a sum of sinusoidal components.

  Approximate Square Wave:

- Use Fourier series to approximate a square wave using a specified number of terms (odd harmonics).

  Visualize Approximation:

- Plot the original square wave and its Fourier series approximations with different numbers of terms to observe how the approximation improves as more terms are added.

  Explore the Convergence of Fourier Series:

- Study how increasing the number of terms in the Fourier series leads to a better approximation of the square wave, especially in terms of the sharp transitions.

Code:

import numpy as np

import matplotlib.pyplot as plt

def fourier_series(x, terms):

if terms < 1:

raise ValueError("Number of terms must be at least 1")

result = x - x

for n in range(1, terms + 1, 2):

result += (4 / (np.pi * n)) * np.sin(n * x)

return result

# Define the original square wave function

def square_wave(x):

return np.where(np.sin(x) >= 0, 1, -1)

# Generate x values

t = np.linspace(-np.pi, np.pi, 400)

# Plot different approximations

plt.figure(figsize=(8, 6))

# Plot the original square wave

plt.plot(t, square_wave(t), label='Original Square Wave', linestyle='--', color='black')

for terms in [1, 3, 5, 9]:

plt.plot(t, fourier_series(t, terms), label=f'{terms} terms')

plt.axhline(0, color='black', linewidth=0.5, linestyle='--')

plt.title('Fourier Series Approximation of a Square Wave')

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.legend()

plt.grid()

plt.show()

Input:

Output:

Purpose:

The purpose of this program is to:

- Approximate a square wave using a Fourier series with different numbers of terms.

- Visualize the approximation of the square wave for different numbers of terms in the Fourier series and compare it to the original square wave.

Experiment No: 08

Tittle: visualizing the real, phase, and magnitude components of the sinc function.

Theory:

e sinc function is defined as:

sinc(x)=sin⁡(πx)πx\text{sinc}(x) = \frac{\sin(\pi x)}{\pi x}sinc(x)=πxsin(πx)​

However, in this case, the function is scaled as sinc(4t)\text{sinc}(4t)sinc(4t), which is a normalized version of the sinc function.

- Real Part: The real part of a complex signal is the actual value of the signal at any given time.

- Phase Part: The phase represents the shift of the signal in time. It can be obtained using the angle of the signal, angle(x)\text{angle}(x)angle(x), which gives the phase of the complex values.

- Magnitude Part: The magnitude represents the absolute value of the signal at any point in time. It provides information about the amplitude of the signal.

Objectives:

  Plot the Real Part of the sinc function to show its time-domain behavior.

  Plot the Phase Part to visualize how the phase varies across time (though, for real signals, this may remain constant or be zero).

  Plot the Magnitude Part to observe the amplitude of the sinc function and its decay with time.

  Understand the sinc function's properties and how it relates to signal processing and frequency response.

Code:

import numpy as np

import matplotlib.pyplot as plt

t = np.arange(-2, 2.01, 0.01)

x = 4 * np.sinc(4 * t)

# Plot real part

plt.figure(figsize=(10, 6))

plt.subplot(3, 1, 1)

plt.plot(t, x)

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.title('Real Part')

plt.grid()

# Plot phase part

plt.subplot(3, 1, 2)

plt.plot(t, np.angle(x))

plt.xlabel('Time')

plt.ylabel('Amplitude')

plt.title('Phase Part')

plt.grid()

# Plot magnitude part

plt.subplot(3, 1, 3)

plt.plot(t, np.abs(x))

plt.ylabel('Amplitude')

plt.title('Magnitude Part')

plt.grid()

plt.tight_layout()

plt.show()

Input:

Output:

Purpose:

The purpose of this program is to:

- Visualize the real part, phase part, and magnitude part of the sinc function xxx.

- Plot these components to understand their characteristics over the given time range.